Thorp 18 Drawing Errata and Plans clarifications

| DWG # Date | Related DWGs | Error Uncertainty Modification Detail | Issue | Comment / Resolution |
| --- | --- | --- | --- | --- |
| 526 | 604, 580 | U | lower left corner JT specs: " #12 drill - 6 holes in -1" drawing shows (in the side view) what appears to be 4 Hi-Shears in the Fwd portion, just aft of -604 firewall, but only spec’s 2 Hi-Shears (upper right corner of dwg) for the roll bar center support. No spec is given for the 4 holes forward. The top view doesn't show the swaged collars for the NAS 178 Hi-Shears - forward. | AN bolts are likely to be a viable alternative to High Shear NAS 178-6-11 (4) and NAS 179-6 collar (4); |
| 526 | 604 | U | dwg shows construction lines for what appears to be a 3/4 x 3/4 x 1/16 2024 T-4 Alum Extrusion vertical against the cockpit side of the firewall. This is not detailed by part no. (for example 604-2, etc.) Reference to -604 (across top of page) again shows what appears to be the same extrusion. | -526 Add -2 Angles 3/4 x3/4 x 1/16 2024 T-4 Aluminum Extrusion. (Ref. to -604 dwg ) |
| 580 | 604, 526 | U | Dwg shows a line of rivets (passenger side) corresponding to the installation of the 526/604 extrusions and a trapezoidal doubler plate (-881 Dwg) on the engine side of the firewall. Firewall Trapezoidal Doubler plate (cut away on -580) Is Not identified or dimensioned | Generally accepted that extrusion thickness is .063, Precise dimensions of plate probably not critical but part should be included. some builders refer to drawing 881 for details |
| 481 28 June 63 |  | Error | The vertical and Horizontal offsets of the centers for the upper left corner radii and the large radii for the fuel tank. The horizontal offset of 1" is incorrect. The vertical offset is absent |  |
| 489 10 Oct 63 |  | D | -Add angle from vertical for axis of riser tube (489-2) of 7 deg 10 min (7 -10') |  |
| 509 8 Mar 64 |  | D | Add Note to reflect the B-502 dwg increase in size of lug attach rivets to AN 470 AD5 which increases drill size from #30 to #21. | Noted in Eklund errata 15 Feb 1995 |
| 569 | 568 | D | Rib is pictured, but not identified on the 569 Dwg. |  |
| A 576 | A 582 |  | identified as -582 Correct by inserting "A-" before reference to "582 Doubler" |  |
| A 576 | A 582 |  | Omitted a ½" dimension on Rt side of A-582 Doubler. Plate |  |
| A 576 | A 582 | U | A-576 isn't consistent with A-582. It shows a double row of five rivets, (and omits the trim plate screw holes) where A-582 shows only a single row and A-582 provides for the trim system. | Also noted in Eklund errata 15 Feb 1995 Delete rivet holes on 576 and add trim holes from A 582 onto A 576 |
| 580 |  |  |  | I.D.’d as 881 in Ray Moore’s Drawing Breakdown and Parts Status List at pg 6 |
| B 593 | 527, d-580 |  | Add notes to coordinate in related spots with -527 and D-580 |  |
| C-595 |  |  | Add (optional) C-502 Horizontal Tail. |  |
| 611 |  |  | Next Assy C-502 |  |
| 612 |  |  | Add addn’l rivet holes reflecting the C-502-9 Bulkhead; Next Assy C-502 |  |
| 613 |  |  | Next Assy C-502 |  |
| 632 |  |  | Change 119 AN 526AD-4-3 rivets to AN426AD-4-3 [ Lower L.H. Corner of Dwg] |  |
| 598-1 | 669 | Err | lower RH corner, between the side and top view is a note to "Joggle .025 - 4 Places" and an arrow pointing to the upper, side corner (the 598 is on its side with top to the right). This note reflects 4 joggles with the original high deck, before the 669 cut down deck was incorporated. | The note, as drawn should reflect 2 .025 joggles. |
| A-650 |  |  | Details Two -12 parts. (L Side) corner spacer for mounting bearings (UR) quadrant -12 seal [name one part -13?] |  |
| 686 |  |  | Next Ass’y A-701 | Reference should be to the A revision |
| 689 |  |  | Next Ass’y A-701 |  |
| 694 |  |  | Next Ass’y A-701 |  |
| 704 | A-701 | D | Calls out two #40 drill holes to be done on assembly. Plan A- 701 calls for 3/32x1/2 roll pins 2 req’d. through the nuts. Looks easy to miss the callout and not install roll pins allowing the nut to loosen later. | Reminder to double check jack screws for the roll pins. |
| 733 |  | U | Diameter of the -8 part not specified (Mount spacer tube 1.62 OD x .120 wall is 1.390 ID) | Assume diameter =1.375 |
| A-740 1 Jun 66 |  |  | 2 references to a -3 part, the first is in the lower right and is labeled a "clip" the second in top center and is labeled a "bracket." It should be labeled as -1 | Noted in Eklund correction dated9 June 1998 |
| 792 |  |  | 792-1 Weld Flange - SW 319 Tank Sender mounting Note: Current Stewart Warner D-391 specifications (11-2009) provide for a minimum tank opening of 1.63" D instead of 1.34" D shown in in 792-1. This change dictates a change in the AN-6227 O-Ring and slot dimensions in 792-1. The AN-6227-27 O-Ring equivalent is MS28775-222 | The current D-391 flange hole pattern is not symmetrical. Sender replacement dictates replacement with A-792-1. [Yet to be drawn] |
| 866 |  | D | AN741T12 superceded by MS 27405 T12 (unplated. Plated is T12P) |  |
| 480 |  | E | To use a prefabricated finger strainer with 3/8 pipe threads use AN867-3 welding flange Eklund correction 15/ Feb/1995 | Use AN867-3 welding flange as alternative to part 480 |
| A 521 |  | E | Tab Linkage AN500-8-6-Screw should read AN501-A10-6 or AN502-10-6 Screw Published in T18 Newsletter #48 pg 4 Nov 1979 These 10-32 screws attach the 513 ARM to the A-517L -5 rib on the stabilator anti servo | Eklund correction 15/ Feb/1995 |
| 703 |  |  | Torque Tube -1 LUG detail in upper left corner should read -3 LUG |  |
| A-591 |  |  | “O” Drill should read “V” drill | Eklund correction 7/July/1997 |
| A-650 |  |  | Both the spacer and canopy seal are designated -12. The seal would logically be -18 | Eklund correction 9/June/1998 |
| A-740 |  |  | -3 Bracket–Upper in top view should be -1 Bracket-Upper | Eklund correction 9/June/1998 |
| 550 | 537 |  | The phantom view 537 Beam(REF.) on the lower left side depicts an incorrect Hi-Shear rivet improperly oriented . Drawing 537 Main Beam Inner Wing has the correct Hi_Shear callout | Eklund correction 9/June/1998 |
| 505 | 857L |  | Aft Tunnel the 857L drawing removes the -3 seat angles as well as the 580 seat angles |  |